| FORTA® | Pheno Seal® | FACT-DATA© |
| --- | --- | --- |
| MANUFACTURER FORTA Corporation, 100 FORTA Drive, Grove City, PA, U.S.A. 16127-6399 TELEPHONE: 1-800-245-0306 (724) 458-5221: FACSIMILE: (724) 458-8331 GENERAL DESCRIPTION Pheno Seal® is a chemically stable thermoset laminate used to correct lost circulation in all water, oil, and synthetic-based fluids and cements. Pheno Seale is ground from laminate sheets into flakes of various sizes, which allow for maximum sealability at a very efficient dosage per barrel. APPLICATIONS Pheno Seal® is used in all water, oil and synthetic drilling fluids as a premium-performance Lost Circulation Material (LCM). Pheno Seal® maintains the best cost-to-benefit ratio of any LCM product, greatly outperforming nutshells, fibers, mica and other blended materials for sealing fractures. Pheno Seal® can be used in any fluid-based drilling system, anywhere that significant fluid loss is a major concern. INSTALLATION Pheno Seal® should be added at a rate of 10 to 30 ppb to a 25 to 50 barrel pill, mixed, and pumped downhole. Pheno Seal® Fine and/or Medium gradations may be used, depending on the degree of lost circulation. Pheno Seal® Coarse may be needed in severe cases of lost returns. After regaining circulation, 40 to 160 lbs. per hour (1 to 4 sacks) of Pheno Seal® should be mixed to reduce fracturing of a newly drilled formation. To minimize seepage loss, 80 to 320 lbs. per hour (2 to 8 sacks) of Pheno Seal Fine should be mixed into the system while drilling. PHYSICAL PROPERTIES. Material:; Ground thermoset laminate; Color :; Varied. Form:; Hard, flat chip; Chemical Resistance:; Inert. Specific Gravity:; 1.3; Temperature range; Stable up to 500' F. Length:. AVAILABILITY Pheno Seal®is available to the drilling industry through an authorized FORTA® products distributor. Contact FORTA Corporation for product availability information (1-800-245-0306). PACKAGING Bags 40 lbs. (18.15 kg.) Pallets 50 bags; 2000 lbs. (909 kg.) WARRANTY FORTA® Pheno Seal® is warranted to be free of defects and to meet all quality control standards set by the manufacturer. FORTA® has no control over the placement or use of the fluids in which Pheno Seal® is incorporated, therefore FORTA Corporation assumes no responsibility for the end product. FORTA® and Pheno Seal® are registered trademarks of the FORTA Corporation |